# SIMATS ENGINEERING

### ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0706

COURSE OUTCOME COVERED

CO5:

Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3	Total Marks:30

Annexure A

## Constraint-Based Problem Solving

Code of Conduct:

I RUCHITA.S (192572081) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUCHITA.S

Annexure A

## Constraint-Based Problem Solving

- A multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

SOLUTION:

### PROBLEM UNDERSTANDING

The company is opening a branch in a remote rural location where high-speed wired Internet is unavailable. The network must support cloud applications, video conferencing, and secure communication with headquarters. The solution must also be affordable and reliable.

### CONSTRAINTS

| Constraint | Requirement |
| --- | --- |
| Location | Remote rural area |
| Internet | High-speed wired Internet unavailable |
| Applications | Cloud applications and video conferencing |
| Security | Secure connection to headquarters |
| Budget | Limited |
| Reliability | Uninterrupted operation |
| Scalability | Should support future expansion |

### RECOMMENDED SOLUTION

Internet Connection:
Use a 4G/5G wireless broadband connection as the primary Internet connection. If cellular coverage is unreliable, use satellite Internet as an alternative.

Networking Devices:

- 4G/5G router

- Managed network switch

- Wireless access point

- Firewall/router

- UPS for network devices

Security:

- Site-to-site VPN between branch and headquarters

- Firewall

- WPA3 for Wi-Fi

- Strong user authentication

- Network segmentation using VLANs

NETWORK DIAGRAM:

### JUSTIFICATION

1. Bandwidth:
4G/5G provides sufficient bandwidth for cloud applications and video conferencing where wired broadband is unavailable.

2. Reliability:
A backup connection such as satellite or a secondary cellular connection can be used if the primary connection fails.

3. Cost:
A wireless broadband solution avoids the high cost of installing wired infrastructure in a remote location.

4. Security:
VPN encryption protects communication between the branch and headquarters. A firewall prevents unauthorized access.

5. Future Scalability:
A managed switch and scalable wireless network allow additional users and devices to be added later.

### CONCLUSION

A 4G/5G-based network with firewall, VPN, managed switch, Wi-Fi access point, and backup connectivity provides a practical balance between bandwidth, reliability, cost, security, and scalability.

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

SOLUTION:

### PROBLEM UNDERSTANDING

The university needs campus-wide wireless access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The main challenges are high simultaneous-user demand, interference, security, limited access points, and budget constraints.

### CONSTRAINTS

| Constraint | Requirement |
| --- | --- |
| Users | 3,000+ students |
| Coverage | Classrooms, labs, libraries, hostels |
| Capacity | Many simultaneous users |
| Interference | Must be minimized |
| Security | Secure authentication |
| APs | Limited number |
| Budget | Cost-effective design |

### RECOMMENDED DESIGN

Use a centrally managed Wi-Fi network with strategically positioned high-capacity access points.

### Access Point Placement

Access points should be placed in:

- Classrooms → one or more APs based on user density

- Laboratories → high-capacity APs

- Libraries → APs positioned to cover study areas

- Hostels → APs distributed floor-wise

- Common areas → APs positioned to cover high-user-density zones

Instead of simply maximizing coverage, AP placement should focus on user density and capacity.

### Frequency Band Selection

| Band | Use |
| --- | --- |
| 2.4 GHz | Longer range and better wall penetration |
| 5 GHz | Higher performance and more available channels |
| 6 GHz | Suitable for newer compatible devices and high-capacity environments |

Use 5 GHz/6 GHz for high-density areas whenever compatible devices are available, while 2.4 GHz can provide additional coverage.

### Authentication

Use:

- WPA3-Enterprise

- 802.1X authentication

- Central authentication server such as RADIUS

- Individual student credentials

### NETWORK DIAGRAM

### Interference Reduction

- Use proper channel planning.

- Avoid overlapping channels where possible.

- Prefer 5 GHz/6 GHz in high-density locations.

- Adjust AP transmit power.

- Avoid placing too many APs close together.

- Monitor channel utilization.

### Balancing the Constraints

Coverage: Strategic AP placement covers required buildings.

Performance: High-capacity APs and 5/6 GHz reduce congestion.

Security: WPA3-Enterprise and individual authentication prevent unauthorized access.

Cost: Instead of installing a very large number of APs, high-capacity APs are strategically positioned according to user density.

### CONCLUSION

A centrally managed, high-density wireless network with strategic AP placement, appropriate frequency-band selection, channel planning, and WPA3-Enterprise authentication provides a good balance between coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

SOLUTION:

### PROBLEM UNDERSTANDING

The financial institution needs stronger cybersecurity while employees still need access to sensitive customer information and online banking services. The design must satisfy security, performance, compliance, and budget constraints.

### CONSTRAINTS

| Constraint | Requirement |
| --- | --- |
| Security | Very high |
| Sensitive data | Must be protected |
| Performance | Should not be significantly reduced |
| Compliance | Must follow strict security policies |
| Cost | Operational cost should remain controlled |
| Access | Employees need controlled access |

### NETWORK DIAGRAM:

### 1. Network Segmentation

Divide the network into separate VLANs:

- User VLAN – employee computers

- Server VLAN – application servers

- Finance VLAN – sensitive financial systems

- Management VLAN – network administration

- Guest VLAN – external/guest users

This prevents an attacker who compromises one section from easily accessing the entire network.

### 2. Firewall Deployment

Use firewalls:

- At the Internet gateway

- Between external/DMZ and internal networks

- Between sensitive internal network segments where required

Firewall rules should allow only necessary traffic.

### 3. Authentication

Use:

- Multi-factor authentication (MFA)

- Strong passwords

- Role-based access control

- 802.1X for network access

- Individual user accounts

Employees should receive only the permissions required for their jobs.

### 4. Intrusion Detection and Prevention

Deploy IDS/IPS to:

- Monitor suspicious traffic

- Detect attacks

- Identify unusual network behavior

- Block known malicious traffic where IPS prevention is enabled

- Generate security alerts

### 5. Data Protection

Sensitive customer information should be protected using:

- Encryption

- Secure protocols such as HTTPS/TLS

- Access control

- Regular backups

- Secure database access

## CONSTRAINT-BASED JUSTIFICATION

### Security

Segmentation, firewalls, MFA, and IDS/IPS provide multiple layers of protection.

### Performance

Traffic can be controlled using VLANs and firewall rules without routing all internal traffic through unnecessary security bottlenecks.

### Compliance

Access control, authentication, monitoring, and logging help the organization enforce strict security policies and maintain audit records.

### Budget

Use centralized security management and existing network infrastructure where possible instead of deploying unnecessary hardware at every location.

### CONCLUSION

The recommended architecture uses network segmentation, layered firewalls, MFA, role-based access control, and IDS/IPS. This provides strong protection for sensitive financial information while maintaining acceptable performance and controlling operational costs.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |