## SIMATS ENGINEERING

### ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0906

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

# Scenario-Based

Code of Conduct:

I RUCHITA.S(192572081) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

RUCHITA.S (192572081)

Annexure A

# Scenario-Based

- A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25 School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

  - Determine	the	network	address	and	broadcast	address	for	each department.

  - Identify the valid host IP address range for each subnet.

  - Calculate the number of usable host addresses available in each subnet.

  - Determine the remaining unused IP address range within the given network block.

SOLUTION:

The Smart Campus has been assigned the IPv4 network 192.168.100.0/24. Different departments require different subnet sizes based on the number of devices. By using subnetting, the available IP addresses are divided efficiently while avoiding address wastage.

### 1. Network Address and Broadcast Address

School of Computing (/25)

- Network Address: 192.168.100.0

- Broadcast Address: 192.168.100.127

School of Electronics (/26)

- Network Address: 192.168.100.128

- Broadcast Address: 192.168.100.191

School of Mechanical Engineering (/27)

- Network Address: 192.168.100.192

- Broadcast Address: 192.168.100.223

The network address identifies the subnet, while the broadcast address is used to send data to all devices within that subnet.

### 2. Valid Host IP Address Range

- School of Computing: 192.168.100.1 – 192.168.100.126

- School of Electronics: 192.168.100.129 – 192.168.100.190

- School of Mechanical Engineering: 192.168.100.193 – 192.168.100.222

These addresses can be assigned to computers, printers, servers, and other network devices.

### 3. Number of Usable Host Addresses

- /25: 128 total addresses → 126 usable hosts

- /26: 64 total addresses → 62 usable hosts

- /27: 32 total addresses → 30 usable hosts

The first address is reserved as the network address and the last address is reserved as the broadcast address.

### 4. Remaining Unused IP Address Range

Unused range:

192.168.100.224 – 192.168.100.255

This unused block contains 32 addresses, which can be allocated later if another department or laboratory is added to the campus network.

### Conclusion

Subnetting helps the Smart Campus allocate IP addresses efficiently. Each department receives enough host addresses according to its requirements, and unused addresses remain available for future network expansion.

- A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24 Quality Assurance → /25 Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

  - Determine the network address and broadcast address for each department.

  - Identify the valid range of host IP addresses.

  - Calculate the number of usable hosts in each subnet.

  - Identify the unused IP address range remaining within the allocated IP block.

SOLUTION:

The company has received the private IP block 10.10.0.0/16. Different departments require different subnet sizes. Proper subnetting ensures efficient IP address management and improves network performance.

### 1. Network Address and Broadcast Address

Software Development (/24)

- Network Address: 10.10.0.0

- Broadcast Address: 10.10.0.255

Quality Assurance (/25)

- Network Address: 10.10.1.0

- Broadcast Address: 10.10.1.127

Human Resources (/26)

- Network Address: 10.10.1.128

- Broadcast Address: 10.10.1.191

Executive Management (/27)

- Network Address: 10.10.1.192

- Broadcast Address: 10.10.1.223

### 2. Valid Host Address Range

- Software Development: 10.10.0.1 – 10.10.0.254

- Quality Assurance: 10.10.1.1 – 10.10.1.126

- Human Resources: 10.10.1.129 – 10.10.1.190

- Executive Management: 10.10.1.193 – 10.10.1.222

These IP addresses can be assigned to employee computers, printers, and other networking devices.

### 3. Number of Usable Hosts

- /24: 254 hosts

- /25: 126 hosts

- /26: 62 hosts

- /27: 30 hosts

Each subnet is designed according to the number of users in the department.

### 4. Remaining Unused IP Address Range

Unused addresses begin from

10.10.1.224 to 10.10.255.255

This large unused address space allows future expansion of departments, servers, or branch offices.

### Conclusion

Subnetting enables the company to organize departments into separate logical networks. This improves security, simplifies network management, and provides room for future growth.

- A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network: 2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

  - Convert the given IPv6 address into its compressed notation.

  - Expand	the	compressed	address	back	into	its	full	hexadecimal representation.

  - Identify the network prefix using the /64 prefix length.

  - Calculate the total number of host addresses supported within the subnet.

  - the total number of possible host addresses available within this network.

SOLUTION:

The hospital has been assigned the IPv6 address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

IPv6 provides a much larger address space than IPv4, making it suitable for hospitals with thousands of connected medical devices and IoT sensors.

### 1. Compressed IPv6 Address

2001:db8:abcd::/64

Leading zeros are removed and consecutive groups of zeros are replaced with "::".

### 2. Expanded IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

The compressed notation is expanded by restoring all omitted zeros.

### 3. Network Prefix

Using the /64 prefix, the network portion is

2001:0db8:abcd:0000::/64

The first 64 bits identify the network, while the remaining 64 bits identify the host or interface.

### 4. Number of Host Bits

Host bits

= 128 − 64

= 64 bits

### 5. Total Host Addresses

264=18,446,744,073,709,551,6162^{64}=18,446,744,073,709,551,616264=18,446,744,073,709,551,616

Therefore, this IPv6 subnet supports 18,446,744,073,709,551,616 unique host addresses.

### Conclusion

IPv6 provides an enormous number of addresses, ensuring that every medical device, sensor, and computer in the hospital can receive a unique IP address without running out of address space.

- An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries: 192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

  - The destination subnet to which the packet belongs.

### The matching routing table entry.

  - The next-hop network/interface used to forward the packet.

  - The default route that should be used if the destination does not match any routing table entry.

SOLUTION:

The ISP router contains the following routing table entries:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

A packet arrives with destination IP

192.168.2.45

### 1. Destination Subnet

The destination IP belongs to

192.168.2.0/24

because the first three octets match the subnet address.

### 2. Matching Routing Table Entry

The router matches the entry

192.168.2.0/24

using the routing table.

### 3. Next-Hop Network or Interface

The packet is forwarded through the router interface connected to the

192.168.2.0/24 network.

The router uses this interface to deliver the packet to the correct destination network.

### 4. Default Route

If no routing table entry matches the destination IP address, the router forwards the packet using the default route

0.0.0.0/0

The default route sends unknown traffic to another router or gateway, which attempts to find the correct destination.

### Conclusion

Routers examine the destination IP address, compare it with the routing table, and forward packets using the best matching route. If no match exists, the default route is used.

- A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

    - Determine the network address and broadcast address for each LAN.

    - Identify the valid host IP address range.

    - Suggest suitable IP addresses for three computers in each LAN.

    - Identify the default gateway that hosts in each LAN should use for communication.

SOLUTION:

The company has two LANs connected by a Layer-3 router.

### Administration LAN

Subnet: 192.168.10.0/26

Network Address

- 192.168.10.0

Broadcast Address

- 192.168.10.63

Valid Host Range

- 192.168.10.1 – 192.168.10.62

Three Sample Computer IP Addresses

- PC1 – 192.168.10.2

- PC2 – 192.168.10.3

- PC3 – 192.168.10.4

Default Gateway

- 192.168.10.1

### Finance LAN

Subnet: 192.168.10.64/26

Network Address

- 192.168.10.64

Broadcast Address

- 192.168.10.127

Valid Host Range

- 192.168.10.65 – 192.168.10.126

Three Sample Computer IP Addresses

- PC1 – 192.168.10.66

- PC2 – 192.168.10.67

- PC3 – 192.168.10.68

Default Gateway

- 192.168.10.65

### Conclusion

Each LAN has its own subnet, host range, and default gateway. The Layer-3 router enables communication between the Administration and Finance departments while keeping their broadcast domains separate, resulting in better performance, easier management, and improved network security.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understandin g of the scenario and context | Shows good understandi ng with minor gaps | Partial understandin g; misses key aspects | Misinterpret s or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganize d and difficult to understand |